## JA030R Series Cross Flow Fan

FEATURES

154mm (6.1in) to 366mm (14.4in) lengths. AC shaded type motor, temperature resistant aluminum alloy, ball or sleeve bearings. Maximum Pressure: 0.8mmAq (0.0315inH2O) to 1.7mmAq (0.067inH2O), Maximum Air Flow: 14CFM (0.4m3/min) to 60CFM (1.7m3/min). CE, UL, TUV listed.

DIMINSIONS                                 (size:mm)

Dimensions and Specifications are subject to change without notice,

please refer to the actual product.

SPECIFICATIONS

| Model \ Spec | Size(mm) |  |  | Rated Voltage V | Rated Frequency Hz | Rated Current A | Input Power W | Speed r.p.m | Max Pressure mmAq | Maximun Air Flow m³/min | CFM | Noise dBA | Safety Approval |  |  | Weight g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C |  |  |  |  |  |  |  |  |  | UL | TUV | CE |  |
| JA03009R12 | 98 | 108 | 154 | 120 | 50/60 | 0.11/0.09 | 8/7 | 2500/2900 | 0.8/1.1 | 0.4/0.5 | 14/17 | 21/24 |  |  | O | 390g |
| JA03009R23 |  |  |  | 230 | 50/60 | 0.07/0.05 | 8/7 | 2500/2900 | 0.8/1.1 | 0.4/0.5 | 14/17 | 21/24 |  | O | O |  |
| JA03015R12 | 160 | 170 | 222 | 120 | 50/60 | 0.14/0.12 | 9/8 | 2700/3100 | 0.9/1.2 | 0.7/0.8 | 24/28 | 27/30 | O |  | O | 520g |
| JA03015R23 |  |  |  | 230 | 50/60 | 0.09/0.07 | 10/9 | 2700/3100 | 0.9/1.2 | 0.7/0.8 | 24/28 | 27/30 | O | O | O |  |
| JR03019R12 | 198 | 208 | 260 | 120 | 50/60 | 0.14/0.12 | 9/8 | 2700/3100 | 1.0/1.3 | 0.9/1.0 | 32/35 | 27/30 | O |  | O | 540g |
| JA03019R23 |  |  |  | 230 | 50/60 | 0.09/0.07 | 11/9 | 2700/3100 | 1.0/1.3 | 0.9/1.0 | 32/35 | 27/30 | O | O | O |  |
| JA03029R12 | 300 | 310 | 366 | 120 | 50/60 | 0.20/0.17 | 15/13 | 2700/3100 | 1.4/1.7 | 1.5/1.7 | 53/60 | 32/34 | O |  | O | 670g |
| JA03029R23 |  |  |  | 230 | 50/60 | 0.09/0.07 | 12/11 | 2700/3100 | 1.4/1.7 | 1.5/1.7 | 53/60 | 32/34 | O | O | O |  |

P-Q CURVE